# NitrousEngineSim Program description

## Introduction

In June 1992, UKSEDS’ first inter-university student rocket Aspire1 was launched successfully to supersonic airspeed at an international student rocketry gathering in France. (Its recovery system was less successful!)

Back at Uni in the autumn, UKSEDS members Rick Newlands, James MacFarlane and James Murray, began work on a simple trajectory simulator for future Aspire rockets, as there was no such software available.

Originally coded in Turbo Pascal, this ‘Aspiresim’ has been added to and expanded over the years by Rick Newlands. Ported into Turbo C in 2000, it’s currently coded in C++, and resides within the Microsoft Visual Studio* architecture on Windows machines so that a user interface (GUI) could be added.

‘NitrousEngineSim’ is a cut-down version of the engine simulation part of ‘Aspiresim’, allowing you to simulate the static firings of hybrid or liquid rocket engines using Nitrous oxide as the oxidiser. The fuel choices can either be externally pressurised, or self-pressurised by their own vapour pressure.

A real (as opposed to ideally-expanded) sea-level rocket nozzle is simulated, with attendant flow separation due to too large an expansion ratio should this occur.

To re-run the sim, simply exit the program and re-run it. All the values you entered into the user interface are saved and re-loaded automatically for the next run.

Words in bold are listed in the glossary at the end of the paper.

*Note that despite the confusingly-similar names, ‘Visual Studio’ and ‘Visual Studio Code’ are two different Microsoft products. Currently, both are free. ‘Visual Studio Code’ will be familiar to students coding for microcontrollers such as Arduino boards.

# Part 1: Running the program, the user interface

## Startup

When the program is executed, the first things you’ll see are this main window, a menu on the top of the screen, and the white sub-window labelled Engine Graphs.

Also shown near the middle of the window are two as-yet blank rectangles labelled fuel and oxidiser which will show the diminishing propellant tank levels as the sim runs.

## The Engine Graphs sub-window

The white sub-window with horizontal gridlines running across and up it, will be used to display graph-plots of engine parameters (thrust, pressure etc) with burn time as well as printouts of engine parameters.

The user-inputs on the right (and also below the time axis) allow you to rescale individual graphs to suit the size of engine to be simmed. Once you’ve input/altered the graphs scales, you must click the Update scales button or your changes will be lost.

The 11 graphs that are subsequently plotted are generated by file EngineModelInterface.cpp so read that file to discern or alter what each graph represents.

## The top menu

The program is run by progressing across the top menu from left to right. Currently, only the 1st menu option Setup engine is available for the ‘NitrousEngineSim’ program, and is the first thing you should click when the program starts.

## Engine inputs

Clicking the menu option Setup engine brings up the Engine inputs submenu, which contains a large number of user-input parameters to set-up your static firing simulation.

Firstly, note on the bottom left-hand side a combo-box of selectable pre-loaded engines with the button Load engine next to it. Select the engine you want, then press the Load engine button to load all of the parameters in.

Below the Load engine button is the Save engine button. This lets you save your input data in a user-named file in subdirectory \Nitrous Engine Sim\Nitrous Engine Sim\User_files\Engines. Your file will have the file extension .engine and will appear in the combo-box next time the program is run. This is a rudimentary save function that will overwrite files with the same name – you have been warned!

At the top left-hand side, we have the Engine type box, where you select the fuel and engine type you want to simulate. For example, HDPE is High Density PolyEthylene solid fuel grain for a hybrid, and IPA is (undiluted) Isopropyl Alcohol (Isopropanol) liquid.

I can add extra propellant combinations, but it takes me several hour’s work to assemble new propellant data; go easy on me!

N.B: The type of fuel pressurisation (vapor pressure or externally-applied pressure) is currently tied to the engine/fuel type selected, so this will over-ride your previous entries – best to select this first.

To the right of this is the Nitrous oxide system box, where you specify the initial parameters for the nitrous oxide run-tank, and also the feed system and injectors leading from it to the combustion chamber.

The Set Press and Set Temp radiobuttons allow you to either specify the initial vapour pressure (Bar absolute), or initial bulk temperature (degrees Celcius) of the Nitrous in the tank.

The Ullage percentage is the percentage of tank volume given over as a vapour head-space above the level of the liquid.

The Feed Loss factor describes the irreversible ‘viscous friction’ losses of the feed plumbing between tank and injector manifold, upstream of the injector (described later).

Orifice diameter is the diameter (mm) of each individual oxidiser orifice within the combustion chamber injector.

The K2 discharge coefficient is a standard dynamic pressure/friction loss coefficient per injector orifice (described later). The ‘2’ denotes its position within a typical plumbing static testing rig: tank pressure sensor =1, injector pressure sensor = 2, combustion chamber pressure sensor = 3.

The default value is 2.0 for nitrous oxide which Aspirespace derived from our numerous nitrous hybrid test-firings.

To the right of this is the Hybrid combustion chamber box, where you input data for the solid fuel grain within the combustion chamber of your hybrid engine.

As ‘NitrousEngineSim’ is a cut-down version of ‘Aspiresim’, only cylindrical ports are currently simulated.

Individual port dia and Maximum port dia specify the initial diameter, and maximum allowable extent, of the cylindrical port/s within the fuel grain. All ports in a multi-port are the same size.

Click the radiobutton to choose your multiport configuration.

Solid density is the density of the solid grain in kilogrammes per cubic metre.

Grain length is the length of the fuel grain alone (millimetres), i.e. excluding any spacers or pre/post combustion chambers.

Grain outer diameter is the outer diameter of the solid fuel grain (millimetres). This is also the inner diameter of the combustion chamber insulative liner (which is currently simmed with zero thickness and zero erosion rate).

To the right of these are the solid fuel regression rate equations: the upper radiobutton is for selecting an equation using the oxidiser mass flux as it enters the grain only:

r=a G0n

Where a is a constant, Go is the oxidiser mass flux entering the port, and n is its exponent.

The lower radiobutton equation includes both fuel and oxidiser mass fluxes as they exit the fuel grain

as well as the length of the fuel-grain L:

r=a Go+Gfn Lm

where Go and Gf are the oxidiser and fuel mass fluxes respectively as they exit the fuel port.

Units for the regression rate equations are kilograms, seconds, metres, and square metres.

Below these are the combustion chamber dimensions for the pre-combustion chamber and post-combustion chamber. These can be set to zero if required. These lengths are added to the Grain length above to set the combustion chamber length.

To the right of this is the Biprop fuel system box, where you specify the initial parameters for the bipropellant fuel’s run tank, and also the feed system and injectors leading from it to the combustion chamber.

This is essentially the same as the Nitrous oxide system box, but with a few extra parameters:

External pressure – and its associated Ext. press radiobutton – allow a liquid fuel with negligible vapour pressure such as alcohol, to be pressurised by an external inert gas source (e.g. nitrogen), regulated to a constant pressure.

The liquid fuel’s density is input as Bulk density.

Alternatively - if you’re feeling brave – the Shared press radiobutton allows such a liquid fuel to be pressurised by the Nitrous vapour in the oxidiser run tank via a cross-feed and bladder/piston.

Note: I’ve only just enhanced the sim to accommodate such cross-feed ‘bravery’, so I need test-firing data from you to validate this enhancement: this mod is as-yet unverified!

Finally, below all of these is the Nozzle box for inputting the nozzle parameters.

Throat erosion rate is the inevitable widening of the throat diameter with burn time (millimetres per second) of an ablative nozzle, such as one employing graphite at the throat. For non-ablative nozzles, this can be set to zero.

Area ratio is the standard ratio of throat cross-sectional area to nozzle exit cross-sectional area. This is the pre-firing ratio before it ablates during the burn.

Note that the Efficiency is a combined efficiency of both the combustion efficiency (C* efficiency) and the nozzle (CF efficiency) i.e:

Thrust=E  C* Cf =E Ce

where E is this efficiency, C* is the combustion’s Characteristic velocity, and CF is the nozzle’s Thrust coefficient. Ce is the engine’s Effective exhaust velocity.

Once all the input data have been entered, press the OK button to load it into the sim. This also writes the input data into a file allowing the sim to remember what was typed in the last time the program was run.

## Running the static firing simulation

Only once the data has been input, a rather self-explanatory Fire button will appear!

Various engine parameters will now appear on the engine graphs window, including how much propellant was burnt, the Total impulse of the firing (along with the UKRA motor code letter designation), and the average (frozen) Specific Impulse.

The test data from this simulated static firing will appear as a comma-separated-values text file Firing_data.csv in subdirectory \Nitrous Engine Sim\Nitrous Engine Sim\User_files\Output_files

A gamut of engine parameters are written to this text file every 0.02 seconds (50 Hz) during the firing.

Also included in this subdirectory is an example Excel spreadsheet that you can copy-and-paste this firing data into: open Firing_data.csv in Excel, then copy it over.

N.B. If Excel still has control over this file when you next run the sim, it’ll get upset! Be sure to close the .csv file as soon as you’ve copied its contents.

After the firing ends, click OK to leave the sim.

## Hints for using the program: Tuning up a hybrid

You’ve simmed a s hybrid motor, but it isn’t performing as required.

- To increase the thrust while keeping the combustion chamber pressure and mixture ratio the same: increase both the nozzle throat diameter and fuel port/s diameter by the square root of the thrust increase factor (e.g. 1.2 times the current thrust) that you want, while simultaneously increasing the number of injector orifices by the thrust increase factor. Don’t forget to increase the fuel grain length and outer diameter.

- To lower the chamber pressure to increase the injector pressure drop: increase the nozzle throat diameter (note that this will increase the thrust, but lower the ISP). Remember to simultaneously reduce the number of injector orifices, or the port mass flux will increase.

- To tune the average mixture ratio, vary the fuel port length.

Note that the peak hybrid fuel port mass flux is printed on the screen, but this is the maximum that occurs on motor startup so is only a guide to the order of magnitude.

The correct peak flux to check is that which occurs when the engine has reached about 85 percent of its initial chamber operating pressure, a second or so after startup. You can get this from the graphs in the engine data spreadsheet: the plotted mass flux graph has a definite ‘knee’ in its curve at this point. Aspirespace’s highest successful flux at this knee point was 600 kg/m2s – higher flux than this “blows the candle out”.

Known software issues:

If the combustion chamber fuel-to-oxidiser ratio is way off, the program can ‘bounce off’ the limits of the data tables, which can cause wyrd effects such as the vapour phase going on forever without draining the tank further. Quit the program, and change your injector inputs:

# Part 2: software description

## Software choice

NitrousEngineSim currently sits within Microsoft’s Visual Studio C++ framework (formerly known as MSVC). As ever, the reason is historical – this was the system I was taught to use at work. And it’s free.

While Visual Studio has fantastic debugging tools, the phrases “easy to learn”, “user-friendly”, and “reliable” aren’t phrases I associate with Microsoft products, and Visual Studio doesn’t buck my trend.

It would be far better if this sim were ported into something students are currently familiar with - e.g. some flavour of Linux – provided that the sim’s GUI can be recreated. The plan is to put together an informal working group of interested students and myself to achieve this: as the end-users of the software, what code platform would you prefer?

Above all else we need to keep the code readable and as simple as possible, so that it’ll be useful to rocketry students across a wide range of disciplines.

Do bear in mind that I graduated in Aeronautics with Astronautics not computing, so apologies in advance to Software engineering students for my code layout and wobbly terminology,

(Sorry about the C++ polymorphism used to select differing propellants, there’s undoubtedly a simpler way.)

## Variables that remember

Sim programs are time-iterative: program values build upon, or integrate with, the values they had during the previous time iteration.

So it’s imperative that sim variables should remember their previous values when they jump out of sim subroutines, should those subroutines modify their values.

This doesn’t happen unless we declare our variables as static by using the keyword ‘static’ e.g. static bool burnout. It’s good practice to declare all sim variables as static, and then make individual ones non-static (dynamic) if and when required.

All variables within a C++ class are initialised as static variables by default. This makes our lives much easier as it saves having to type ‘static’ before every single variable declaration; it’s easy to miss one!

## Top level software description

The first file called when the program is run is NitrousEngineSim.cpp which does nothing more than interface with the Windows operating system and is essentially the ‘app’.

(.cpp are the file extensions for ‘C plus plus’ i.e. C++ code. Their associated header files have extension .h)

NitrousEngineSim.cpp calls NitrousEngineSimDlg.cpp which is the actual entry point and user interface file for the sim (‘Dlg’ stands for dialog box i.e. the main window).

This file takes input data from the various menu windows and dialog boxes of the Graphical User Interface (GUI) and passes them into the sim via the engines class. There are several ‘Dlg’ files for the GUI, because each dialog box in the user interface requires its own file and header file (it’s a Visual Studio thing).

NitrousEngineSimDlg.cpp file also handles the graphics: a timer is called every 20 milliseconds (50 Hz rate) to redraw the screen and various text outputs.

### Sim Control

This timer also calls Sim Control, which resides within the timer subroutine. Sim Control calls the engine simulation routines via interface file EngineModelInterface.cpp

(Note that ‘Aspiresim’ simulates considerably more than the rocket engine/s, therefore its Sim Control is a separate file SimControl.cpp).

## Files, variables, and data structures of note

- The Cengines class is declared in file Cengines.h and contains essentially all the variables and subroutines used within the engine simulation.

- Useful_functs_and_conversions.cpp contains useful mathematical routines used by several of the program files. Its associated header file Useful_functs_and_conversions.h contains a list of mathematical constants as well as enumerated types used by the program.

- NitrousEngineSim.rc controls all the resources (graphics of the GUI). Edit this file manually at your peril! (One mistake and the whole program crashes: best to use the resource editing wizard in Visual Studio instead.)

- Variable delta_time is the time increment rate that sets how fast the sim runs. It is set within NitrousEngineSimDlg.cpp then exported throughout the sim via pch.h

- Variable simulation_state is directed by Sim Control to Initialise (simulation_state = INIT_SIM), then Run (simulation_state = RUN_SIM), the engine simulation routines.

## Low level software description (the nuts and bolts)

### NitrousEngineSimDlg.cpp

This file is the heart of the graphics outputs and user interface (GUI).

At the top of the file, are all the includes files (#include). These link-in all of the GUI files (one per dialog box) by listing their header files (.h)

For example, pch.h is the standard Visual Studio header file that must be included in all files in the program.

Next we have local variables, structures, and coloured pens for drawing graphics (CPen).

Further down is the Visual Studio’s Message pump’s map of where to go. This is known as ‘message map’, and is demarked by all the commands between the lines BEGIN_MESSAGE_MAP(CNitrousEngineSimDlg, CDialogEx) and END_MESSAGE_MAP

When you click on a dialog box button or menu button, this generates a message in the message pump. This pump handles all the commands that performing such actions should cause. It also handles mouse inputs (clicked left button etc).

The message map then tells the pump where each message should be routed to: each command references (listed on its right) a subroutine that is run when the action happens.

For example, the Fire button has the message name IDC_BUTTON_FIRE. When the Fire button is pressed, the message pump consults the message map code line associated with this message name:

ON_BN_CLICKED(IDC_BUTTON_FIRE, &CNitrousEngineSimDlg::OnBnClickedButtonFire)

which tells it to call subroutine CNitrousEngineSimDlg::OnBnClickedButtonFire() which itself contains the code to do something, in this case to set the sim into a Run state.

(The :: symbology means that this OnBnClickedButtonFire() function is a member of the CNitrousEngineSimDlg Class which is described/assigned in header file NitrousEngineSimDlg.h

Each dialog box has its own functions which may have the same names, so this is how they are kept apart.)

CNitrousEngineSimDlg::OnInitDialog()

This subroutine is the first bit of code that is run when the main window appears (after any ‘constructor’), and is used for initialisation. Ignore the first few paragraphs, and go down to the code marked /*** Sim and graphics initialization ***/   This initialises global sim variables, such as setting simulation_state to INIT_SIM (initialisation mode).

Next, the graphics are initialised, then a Device Context (a DC, Visual Studio’s name for a handle to the graphics surface - its area in memory) is set up: CDC *dc = GetDC(); // get the drawing surface

Next, the Timer is set up, to redraw all the graphics and run the sim every 20 milliseconds (50 Hz):

SetTimer(1001, 20, NULL); // kick in timer with ID 1001 every 20 milliseconds (50 Hz)

Next, the code ‘creates an instance of’ (draws) a dialog box with the identifier IDD_GRAPH_DIALOG, which is the identifier of the black window for drawing engine parameters graphs on: GraphDlg.Create(IDD_DIALOG_GRAPH);

Next, the fuel and oxidiser ‘slider’ progress controls are set up: CProgressCtrl  etc…

Finally, the bitmapped drawing of the Fire button – which has the identifier IDB_BITMAP_FIRE - is loaded and stored in (attached to) a variable of type HBITMAP, to be drawn onto the screen when required: LoadBitmap(IDB_BITMAP_FIRE)   etc..

CNitrousEngineSimDlg::OnPaint()

OnPaint() is called every time the background has to be redrawn.

Ignore the first bit of code, it simply draws the icon when the window is minimised.

The main background graphic "Race2space.jpg" is now drawn:

Firstly a device context for the graphics area is set up with CPaintDC dc(this) and then variable m_hDC is set up with this context.

Gdiplus::Graphics graphics(pDC->m_hDC)

Gdiplus is a wobbly Visual Studio graphics add-on which is set-up in pch.h with a sub-Class called Graphics() . It is then used to draw the image, resized to your screen:

graphics.DrawImage()

CNitrousEngineSimDlg::OnTimer()

This subroutine is called every time the timer - initialised previously with the SetTimer() command -runs to zero, i.e. every 20 milliseconds (50 Hz).

Sim Control resides here.

Variable ‘frame_number’ is defined as 50 which sets the number of times the sim is run per graphics redraw (e.g. 50 times 50 Hz = 2500 Hz). Set this lower to slow the sim down for older computers. If this is set too high, then each sim run might not have finished being computed before the next run is triggered, leading to the program crashing.

Next we have a switch() statement whose output depends upon the current sim state (variable simulation_state).

If simulation_state is equal to INIT_SIM (case INIT_SIM) then the sim is initialised by subroutine ResetEngineModel() in file EngineModelInterface.cpp to initialise the rocket engine.

If simulation_state is equal to RUN_SIM (case RUN_SIM) then the sim is run frame_number number of times per call. EngineModelInterface() (also in file EngineModelInterface.cpp) is run to sim the rocket engine (see later).

Variable sim_time is then incremented to increase the elapsed sim time provided the engine hasn’t reached burnout.

Leaving Sim Control, next some text is printed on the screen in blue using the screen_text() call, giving the current values of selected sim variables such as sim-time.

Finally, the fuel and oxidiser sliders (progress controls) are redrawn using the current amounts of fuel and oxidiser.

CNitrousEngineSimDlg::OnEngine()

This subroutine creates and displays the GUI’s dialog box for inputting engine data, and retrieves user-input data from it.

The dialog box is detailed in EngineSetupDlg.cpp and its header file EngineSetupDlg.h

First, a Visual Studio ‘modal dialog box’ is created (a menu window with an OK button) by creating a class dlg of the type CEngineSetupDlg as defined in EngineSetupDlg.h and then creating the window using the call: dlg.DoModal()

This call is then triggered to return user data when that window’s OK button is pressed, using the expanded call  if (dlg.DoModal() == IDOK), in which case lots of data is retrieved from dlg and stored in the engines class.

Now that the data has been loaded-in, the variable engine.have_input_data is set to true, to tell the rest of the sim that valid engine user-input data is now available.

Lastly, now that this data is available, the Fire button can be made available for clicking on.

CNitrousEngineSimDlg::OnBnClickedButtonFire()

When the Fire button has been clicked, this subroutine sets the sim to a Run state simulation_state = RUN_SIM

And also ignites the simulated engine:  engine.ignition = true

## The engine models

### Mathematical basis

Back in the day, Aspirespace used the following textbook (Ref.1) extensively as it was the only one that included hybrid engine design; we bought several copies. (It’s also a much easier read than Sutton.)

Equations contained within that textbook are used and referenced in the comments within the sim code. Refer to this book for details.

This book is scarce these days, but older rocketeers will have a copy.

And my own rocketry textbook (Ref.2 - shameless plug) covers all the same ground, and retails for a paltry 15 quid.

### The engine models subdirectory

For tidiness, the rocket engine simulation lives in the /Engine_models subdirectory.

The engine sim uses variables and subroutines contained within the Cengine data class defined in Cengines.h

An instance engine of this Cengine class is defined near the top of file EngineModelInterface.cpp thusly:

Cengines engine

When the variables and subroutines of engine are subsequently called, they’re separated by a full-stop e.g.

engine.burn_time = (whatever).

When called by subroutines within the Cengine class, the ‘engine’ name to the left of the full-stop can be omitted, as the compiler presumes it:

burn_time = (whatever).

The engine code resides within file engine_core.cpp, which calls generic subroutines from engine_generic.cpp

### Initialisation

As a veteran sim coder, I can’t stress enough how important it is to make absolutely sure that all variables are initialised. If this isn’t done, strange behaviour can occur during the running of the software.

Of more concern, if a run-time bug is caused by an initialisation problem, it’s nearly impossible to trace. For this reason alone, it’s vital to ensure that every variable that will – or even may - be subsequently used, is nailed-down to some pre-set value. Otherwise, they could be born with any random value that was floating around in memory.

Initialisation should occur right at the start. For example, a Constructor is a C++ subroutine that runs first before anything else. That’s where I put the code line to preset everything in the engines class to zero or false:

Cengines::Cengines()  // standard constructor

{

// Reset everything to zero, false

memset(&engine, 0, sizeof(Cengines));

}

Building upon this ‘zeroed’ foundation, relevant variables can now be initialised to non-zero values as and when required; occasionally set to tiny values to avoid subsequent divide-by-zero errors.

Sim Control now calls subroutine ResetEngineModel() in EngineModelInterface.cpp

Only once user-input data is available, this calls subroutine Load_propep_file() to load the relevant file of Propep-generated thermochemistry data into the sim.

### Load_propep_file()

This thermochemistry data was generated from multiple runs of the Propellant Evaluation Program (PROPEP) originally coded by the Martin Marietta Corp. The original PROPEP is showing its age; its descendant CEA is generally used now. However, it’s much easier to add new propellants to the list contained within PROPEP versus CEA.

Annoyingly, the generated PROPEP data is partly in imperial units, which had to be converted to metric by pre-processing software.

Currently, I am using PROPEP version 3, which Dave Cooper wrapped in a Visual Basic front-end. See https://www.rimworld.com/loggerusb/propep3/intro.html

Load_propep_file() reads this data and stores it in a large three-dimensional array named Propep[a][b][c] with dimensions of combustion chamber pressure, fuel-to-oxidiser ratio, and the third dimension being the data described below:

File format

The thermochemistry data for each propellants combination are stored in text files with the file extension .propep and are in the following format:

Firstly, the number and range of the combustion chamber pressures covered are listed. Typically, there are: number of pressure entries (20), lowest pressure (2.5), highest pressure (95 Bar absolute)

PRESSURE_RANGE 20 2.5 95.0

Then the number and range of the fuel-to-oxidiser ratios are listed:

FUEL_OX_RANGE 191 2.5 97.5 97.5 2.5 // lowest fuel percent with highest ox percent, highest fuel percent with lowest ox percent.

Then the data is arranged in large chunks in descending combustion chamber pressure:

DATA_START

****  95                  // the chamber pressure of this ‘chunk’ in Bar absolute.

2.500 97.500          // the percentage of fuel to the percentage of oxidiser.

2309. 145.00 3.441 // combustion chamber temperature in Kelvin, combustion chamber pressure in Bar absolute, and the Specific gas constant for the combustion chamber gas mixture in J kg−1 K−1

The next two lines give the thermochemistry of the flow through the nozzle. The first line of data lists the results assuming frozen flow, the second line is for shifting flow (equilibrium flow).

ISP,   IS EX,   T*,      P*,   C*,  OPT EX, Disp, AoMst,  Te

221.5 1.3162 1994. 5.35 3905.5 64.47 413.1 0.83734 402.

224.5 1.2617 2043. 5.45 4027.0 64.25 418.6 0.86339 419.

Note that only the frozen flow analysis line is used by the simulation, which gives the lower more realistic performance, so the sim is slightly conservative. This is why such a high combined value of C* and nozzle efficiency (98%) is used. (The first entry of the shifting flow analysis line is also stored for reference.)

In the header above, ISP is the ideal (vacuum) Specific Impulse (in seconds for comparison).

The ideal exhaust velocity 'Ve' can be found by multiplying ISP by g.

i.e.  thrust = nozzle_mass_flowrate * 9.81 * ISP

where nozzle_mass_flowrate is in kg/s

IS EX is the Isentropic exponent such that:

P VISEX=constant

for isentropic flow at the nozzle throat, i.e. this is equivalent to the nozzle throat gamma (γ) and can be used instead of gamma.

(The values of IS EX and gamma do not agree exactly only because the throat gas is simulated as a real gas and not a perfect gas.)

The variables T* and P* are nozzle throat temperature (Kelvin) and pressure (Bar, absolute).

C* is the characteristic velocity in m/s.

OPT EX (optimum expansion ratio) is the ratio of the nozzle exit area to nozzle throat area at which exit pressure will equal the specified ambient pressure.

N.B. note that the ambient pressure defined in the sim is ISA sea-level conditions: 101325 Pascals.

Disp (density ISP) is only used for airbreathing rockets, so ignore it.

AoMst is the ratio of nozzle throat area A* (square metres) to the ensuing nozzle mass flow rate mnozzle (kg/s) therefore:

mnozzle=A*AoMst

Te is the flow temperature at the nozzle exit in Kelvin, assuming an ideal (optimum expansion) nozzle.

After loading the PROPEP file, subroutine ResetEngineModel() now calls subroutine Initialise_engine() in file core_engine.cpp which further initialises engine variables.

### Initialise_engine()

Firstly, the status of both propellants are set to commence burning their liquid phases upon engine ignition:

fuel_status = ox_status = BURNING_LIQUID

Ahem, to avoid having to add an extra label, if the engine is a hybrid, then the burning of the fuel grain is also designated as BURNING_LIQUID. This isn’t entirely contrived, as the hybrid fuel does briefly exist in a liquid state as it melts during combustion (or not-so-briefly in wax hybrids!)

With just-subcritical propellants such as nitrous, the possibility exists for combustion to continue burning with vapour after the liquid phase runs out. This is enabled using Boolean flag:

extended_ox_burn_possible = true

Next, the program deals with the user-input K2 injector flow resistance coefficient, converting it into a Loss factor that is computationally easier.

The K2 flow resistance coefficient is a dynamic pressure/friction loss coefficient per injector orifice (described later). The default value is 2.0 for nitrous oxide which Aspirespace gleaned from the results of multiple nitrous hybrid tests.

Loss factor is our overall loss value for both the feed-system and injector combined:

K2 is converted to Loss factor as:

loss factor= K2NA2 ×PASCALS_TO_BAR

Where N is the number of injector orifices (per propellant), A is the cross-sectional area of an individual orifice (m2), and the conversion factor PASCALS_TO_BAR = 10-5

The loss factor is used later on by the injector model.

Then the run-tank of saturated nitrous oxide is set-up, using either the user-selected tank pressure or tank temperature.  In the former, subroutine on_press(P) is used to calculate the corresponding saturated nitrous tank temperature, whilst in the latter, subroutine vapour_pressure_bar(T) calculates saturated vapour pressure based on nitrous temperature. Both of these reside within nitrous_oxide.cpp

The tank temperature is checked and limited to make sure it doesn’t exceed nitrous’ critical temperature of 36.4 degrees C. If it does, program fault flag fault is set (value of 2) as a warning.

Once the tank temperature is known, the initial nitrous liquid and nitrous vapour densities can be calculated, using further functions within file nitrous_oxide.cpp

Knowing these densities, the input ullage volume, and the run-tank tank volume, the volumes of liquid and vapour within the tank can be calculated, and hence their respective masses.

#### If the engine is a hybrid

A constant used in the description of the solid fuel grain regression-rate, is set-up from user-input data:

regression_rate_constant = regression_a * pow(charge_length, regression_m)

i.e.

C=a Lm

Next, the initial solid grain’s physical properties are calculated: areas, volumes, and mass.

#### If the engine is a bipropellant

The first user-choosable fuels are also just-subcritical propellants (e.g. propane) and so are dealt with using the same techniques and subroutines as described above for the run-tank of nitrous oxide.

In contrast, the last selectable fuel/s (e.g. Isopropyl Alcohol) are far below their Critical temperature, so have negligible vapour pressure. They have to be pressurised by a separate gas.

Next, the nozzle throat cross-sectional area is calculated from its diameter.

Finally, the tracking arrays (see below) are set up, with the initial chamber pressure (Bar absolute) set to just over the lowest value in the data file.

### Running the engine model - Simulate_engine()

Subroutine Simulate_engine() is called at run time by subroutine EngineModelInterface().

This is provided that the engine hasn’t burnt out in which case it is no longer simmed. Burnout is generally taken to be when all the liquid has run out, but I’ve added the facility to sim the extra thrust gained from burning of the remaining vapour, (but only for a few propellant types - nitrous oxide, ethane, and propane).

Flaccid integration

Firstly, values from the previous time iteration’s run are saved for comparison with the current time iteration run.

These ‘old’ values are required to make numerical integrations (of engine parameters) robust. Simple high school physics-type integrations such as ‘V = U + at’  are referred to as being ‘non-stiff’ (flaccid?) which laments their habit of numerically exploding to enormous values.

We encounter this one when we try to simulate the effects of opening a parachute during the rocket vehicle’s trajectory. When the ‘chute opens, there’s a massive Drag increase, resulting in a much larger acceleration value compared to earlier time steps. Simple integration throws a wobbler when this happens, which is why we need to use ‘stiffer’ Trapezoidal integration at the very least - subroutine Integrate() within file Useful_functs_and_conversions.cpp

Next, as the oxidiser being simmed is saturated nitrous oxide, subroutine Subcritical_tank() within file engine_generic.cpp is called, which itself calls functions from file nitrous_oxide.cpp

This subroutine was coded based on a blowdown-type model of a run-tank emptying of saturated liquid, devised by Dr Bruce Dunn.

This model is described in Ref.2 and also in Aspirespace technical paper ‘modelling the nitrous run tank emptying’ downloadable from the Aspire website.

This model calls subroutine Injector_model() in file engine_generic.cpp which is coded thus:

#### Injector mass flow rate

We want to calculate the mass flow rate of liquid propellant that will flow through an injector due to a pressure P across it. Directly upstream of the injector face is the pipe section known as the injector manifold.

We start with Bernoulli’s equation (an energy equation which describes pressure as a form of potential energy and flow velocity as kinetic energy) for the flow of liquid from the injector manifold into the injector orifices:

Pmanifold+12ρliquid Vmanifold2=Porifice+12ρliquid Vorifice2=constant

As the liquid leaves the injector orifices, it breaks into droplets without changing pressure, so Porifice  equals combustion chamber pressure.

Substituting for the velocity V of the liquid from a rearrangement of the mass continuity equation:

V=mliquidA ρliquid

where mliquid is the mass flow rate of liquid, A is the cross-sectional area of either the manifold or injector orifice, and ρliquid is the liquid density,

gives:

Porifice-Pmanifold=∆P=ρliquid  mliquid22 ρliquid2KN Aorifice2-1Amanifold2

= mliquid22 ρliquidKN Aorifice2-1Amanifold2

where N is the number of orifices.

Note the inclusion of a ‘flow resistance coefficient’ K, which is related to a traditional discharge coefficient as:

Cdis=1K

As the static pressure of the Nitrous liquid drops as it passes through the orifices, it vaporizes. This means that what flows through the injector is a foam of liquid and bubbles, but mostly vapour, and so traditional tables of loss values or discharge coefficients don’t work for this mixed fluid. You have to tailor this K coefficient until the time taken to empty the tank matches your own test results.

We’ve found that a good starting value for K is 2.0 for Nitrous oxide and other just-subcritical propellants. (Therefore Cdis=1K=12=0.707 )

Interestingly, this value of 2.0 applies to either liquid or vapour, because the liquid vaporizes almost completely into vapour as it flows through the orifices, due to its pressure dropping significantly.

Rearranging, and assuming that Amanifold2  is ‘infinitely’ larger than an injector orifice area gives:

mliquid=NAorifice2 ρliquid ΔPK

Grouping terms in this equation allows the entire injector plate to be described by a ‘Loss factor’, which we’ve found useful for comparison with actual injector flow test data:

mliquid=2 ρliquid ΔPFloss    where   Floss=KN Aorifice2=1Cdis N Aorifice2

In Aspirespace, we denote K as K2, the ‘2’ signifying its position within the plumbing of our typical static testing rigs: tank pressure sensor = 1, injector pressure sensor = 2, combustion chamber pressure sensor = 3.

Having calculated the mass flow rate of liquid nitrous out of the tank, we can integrate it to get the mass that has left the run-tank over each time iteration dt:

Liquid mass mliquid has decreased by:

∆mliquid= mliquiddt

And from the law of mass continuity, the total nitrous mass within the run-tank has also decreased by this amount.

The resulting new value for mliquid is the mass of liquid that would now be in the tank if the nitrous didn’t react to the expansion of the nitrous vapour and the ensuing drop in pressure. We’ll designate this as mliquid_unphased:

mliquid_unphased=mliquid-∆mliquid

But the nitrous does react, both to the increase in nitrous volume and the drop in temperature. We now need to calculate how it responds:

The densities of the liquid and vapour are functions of temperature only, so the nitrous is forced to follow a run-tank volume formula:

Vvapour + Vliquid = Vtank    or:   mliquidρliquid+mvapourρvapour=Vtank

Where ρ is density, m is mass.

The resulting changes in the proportions of the two phases can then be calculated by rearranging this equation:

mliquid=ρliquidVtank-mvapourρvapour

substituting:

mliquid=ρliquidVtank-mtotal - mliquidρvapour

collecting terms:

mliquid1-ρliquidρvapour=ρliquidVtank-mtotalρvapour

dividing by ρliquid:

mliquid1ρliquid-1ρvapour=Vtank-mtotalρvapour

gives:

mliquid=Vtank - mtotalρvapour1ρliquid - 1ρvapour        and    mvapour=mtotal-mliquid

We’ll designate this new volume-adjusted value as mliquid_new.

From mass continuity, the discrepancy between mliquid_new and mliquid_unphased has to be the mass of Nitrous that has been vaporised by the phase changes:

mvaporised = mliquid_unphased - mliquid_new

Now, previous incarnations of the engine sim utilised simple (flaccid) integration rather than Trapezoidal integration and this running mvaporised calculation was unstable. I thus had to stabilise it, with a filter known as a first-order time lag.

Providentially, it transpired that the real-world nitrous vaporisation process isn’t instantaneous, and a time lag of about 0.15 seconds gives a good match with our run-tank test data.

So this time lag code remains:

bob = liquid_mass_unphased - *liquid_mass;

time_constant = delta_time / 0.15;

lagged_bob += time_constant * (bob - lagged_bob);

*running_vaporised_mass = lagged_bob;

Note the second-last line of code within this subroutine which deals with the occasionally occurring program fault that the amount of liquid nitrous in a nearly-empty tank can bounce off zero then increase again. If so, a program fault is flagged (with a value of 9), that forces engine burnout:

if (mliquid_new > mliquid_unphased) then trigger burnout.

Subroutine Subcritical_tank() returns the mass flow rate exiting the tank, as well as the amount of liquid and vapour remaining within the tank.

### Nitrous roulette

In 2025, I added a new precursor subroutine before Subcritical_tank(), attempting to model those bipropellant engines that use the nitrous vapour pressure to pressurise the fuel tank via a (hopefully) impermeable piston or bladder. This subroutine Nitrous_roulette() accounts for the effective rate of increase of the nitrous run-tank volume as a volumetric flow rate of liquid fuel is expelled from its run-tank:

dVdtox_tank=Vfuel    where:   Vfuel=mfuelρfuel        with ρ being density.

This is time integrated to get the current oxidiser run-tank volume, to pass into Subcritical_tank().

Vox_tank=dVdtox_tankdt

I’ll need test data from firings of such biprops to validate this code, though the sim subsequently behaves as one would expect.

#### Vapour-only run-tank emptying

Once all the liquid has drained out of the run-tank, the tank emptying vapour-only subroutine Subcritical_tank_no_liquid() is then called, which itself calls functions from file nitrous_oxide.cpp

Historically, I modelled the vapour-only run-tank emptying process as a classic Isentropic process, but fudged to account for the real gas behaviour of the nitrous vapour; this appeared to work as it matched our test-data very well.

To my relief, Ref. 5 confirms that this approach was valid, and suggests new parameters to generalise the isentropic parameter γ, albeit requiring hard-to-obtain thermophysical data.

My pragmatic approach involved data that wI did have, such as Nitrous oxide’s Compressibility factor (Z). Its graph is shown here:

The nitrous vapour travels up the blue-coloured saturated vapour line as it empties from the run-tank and loses pressure.

We can approximate this vapour curve as the straight line (coloured red) running from the Critical point up to the point (0,1) at zero pressure.

Thus, the compressibility factor is modelled essentially as a linear function of tank vapour pressure.

The subroutine begins the vapour-only phase-emptying model by snapshotting the compressibility factor just as the liquid phase has run out.

Then the mass flow rate of nitrous vapour out of the tank is calculated as was done in subroutine Subcritical_tank(), but working with vapour instead of liquid. The loss coefficient K remains the same though.

Both the nitrous mass within the run-tank, and the run-tank internal volume, are known at each time iteration, so their quotient is now calculated, which is the current nitrous vapour density:

ρnitrous=mnitrousVrun-tank

Next, the vapour emptying can be approximated as an Isentropic process. The inter-relationships between P, T, and  for an isentropic process are the standard Isentropic equations:

dTT=γ-1γ dPP=γ-1dρρ     or:     T2T1=P2P1γ-1γ=ρ2ρ1γ-1

where γ is the ratio of specific heats which has value 1.3 for nitrous vapour (averaged over the subcritical temperature range of interest).

The code takes state 1 as the ‘old’ values from the previous time iteration, with state 2 the current time iteration.

Starting with the for a real gas equation P=ZρRT

Then:

P2P1=Z2Z1ρ2ρ1R2R1T2T1   where R is the specific gas constant for nitrous.

We assume that R doesn’t notably change with time, then R2 = R1 and that term drops out.

Using the above isentropic relationships to substitute density for temperature gives:

P2P1=Z2Z1ρ2ρ1ρ2ρ1γ-1= Z2Z1ρ2ρ1γ

Rearranging the real gas equation for temperature similarly gives:

T2T1=P2P1-1Z2Z1-1ρ2ρ1-1=ρ2ρ1γZ2Z1-1ρ2ρ1-1=Z2Z1-1ρ2ρ1γ-1

There is one problem though: the code needs the value of Z2 in the above equations, but this depends upon P2, the vapour pressure at time 2, which gets calculated after calculating these equations.

So we need to resort to recursion: a ‘running’ Z2 is continuously re-calculated based on the previous time iteration’s tank pressure.

#### If the engine is a hybrid

Now that the oxidiser tank has been modelled, the fuel grain port’s regression is now modelled by subroutine Hybrid_grain_erosion()

Firstly, this subroutine checks that the port/s physically still exist (non-zero areas).

Then the mass flux G (per port) is calculated as:  G=mA  where A is the port cross-sectional area and m is the port mass flow rate.

Next, the form of the regression equation that was user-input is checked:

If it was based upon the incoming liquid mass flux only, then the port fuel mass flow rate equals:

msolid=ρsolid  r Asurface

where r  is the regression rate and Asurface is the port internal surface area.

However, if the regression equation was based upon the combined fuel and oxidizer flux at the nozzle end of the port, then a recursive loop is required as described in Humble, bottom of page 423, and repeated here:

- Start by assuming that the total mass flux is just the liquid flux (i.e. the gaseous flux caused by the liquid having passed through the injector and vapourised, then travelling down the solid port.

- Determine the regression rate of the solid grain and ensuing mass flow rate based on this mass flux.

- Add the solid and liquid fluxes (by which I mean the gaseous mass fluxes caused by the liquid and solid propellants) to get a new mass flux.

- If the old guess at the total mass flux is very nearly equal to the new flux then the recursion can finish.

- If not, go back to step 2. And iterate with the new total flux.

Step 4. Is coded as requiring the old and new fluxes to be within 1% of each other.

Then, after storing the peak mass flux for screen output, the propellant mixture ratio is calculated as the ratio of fuel to oxidizer mass flow rates at the end of the port.

The program then leaves subroutine Hybrid_grain_erosion() and returns.

Next, the rate of change of combustion chamber volume is calculated, to be used later. The volume increases as the fuel grain port/s erode.

The rate at which each fuel port surface erodes is:

raverage S    where  S=2π raverage L   is the port surface area (assuming a circular port).

The chain rule of differentiation shows that this term (summed over all ports) is also equal to the time rate of change of chamber volume, or the volumetric flow rate (mass flow rate divided by gas density) of gasified fuel.

dVdt=dVdr drdt=ddrπ raverage2 L  raverage=2π raverage L  raverage= raverage S

Note that for a bipropellant engine, this dVdt = 0

#### If the engine is a bipropellant

In this case, if the chosen fuel is a just-subcritical propellant then subroutines Subcritical_tank() and Subcritical_tank_no_liquid() are called in exactly the same manner as was done for the nitrous oxide run-tank.

If however, the chosen selectable fuel is a liquid with negligible vapour pressure (e.g. Isopropyl Alcohol) then it has to be pressurised by a separate gas. This can either be a simulated inert gas regulated at a constant feed pressure, or (heaven forfend) the vapour pressure of the nitrous oxide.

Subroutine Externally_pressurised_tank() is then called. Similarly to subroutines Subcritical_tank() and Subcritical_tank_no_liquid(), it uses the Injector_model() to model the outflow from the fuel run-tank.

#### Combustion simulation

With the mass flow rates calculated, the combustion can now be modelled:

The combustion chamber mixture ratio is calculated as the fuel mass flow rate divided by the oxidiser mass flow rate.

The two flow rates are summed to obtain the total mass flow rate heading for the nozzle (variable total_inflow).

Next comes the array tracker:

This is a section of code that matches the current fuel-to-oxidiser ratio, and current combustion chamber pressure, with the correct section of the three-dimensional array of PROPEP combustion chamber data. It does this by keeping track of the changing oxidiser ratio and combustion chamber pressure and increasing or decreasing the 3D array indexes as required.

Checks are performed to ensure that the fuel-to-oxidiser ratio and combustion chamber pressure don’t wander off of the limits of the array.

Once within the correct portion of the array, combustion output parameters can be obtained by double-linearly-interpolating between both pairs of fuel-to-oxidiser ratio and combustion chamber pressure. This is performed by function DoubleInterpolate(). For example, the combustion gas temperature (chamber temperature) is found by double-interpolating with parameter number 10 of the 3rd dimension of the array:

chamber_temperature_K = DoubleInterp(10)

Similarly, the Specific gas constant for chamber gas mixture (J/kg K) can be obtained from parameter number 12 of the 3rd dimension of the array, with C* obtained from parameter number 4.

The rocket nozzle

Nozzle throat conditions P*, throat isentropic exponent (equivalent to gamma), and AoMst are similarly obtained. Recall that AoMst (as stored in variable throat_area_per_mdot) is the ratio of nozzle throat area A* (square metres) to the ensuing nozzle mass flow rate. With these values the nozzle can now be simulated.

Firstly, the nozzle mass flow rate is calculated from the above throat_area_per_mdot and the nozzle throat cross-sectional area (calculated from the user-input throat diameter).

This nozzle mass flow rate is then passed through a first-order time lag of 0.1 seconds: this is necessary to prevent numerical oscillations in mass flow rate as can occur in real rocket engines.

Next, the chamber stagnation pressure P0 is calculated from C* as a program check just in case this deviates from the PROPEP input chamber pressure (it doesn’t as it happens).

A standard isentropic flow relation (Humble equation 3.95 page 101) is now used to find the throat Stagnation pressure P0* from the PROPEP-given throat Static pressure P*

P0*= P*1+γ-12M2γγ-1   assuming that the throat Mach number M is 1.0

[   At this point a Program check was performed to see if PROPEP’s given chamber pressure P, chamber stagnation pressure P0, and throat stagnation pressure P0* were pretty much equal, and indeed they were, as verified during debugging runs, which verifies the frozen-flow assumption of constant stagnation pressure along the nozzle, and the assumption that the throat Mach number is 1.0

Despite being called a 'Frozen' analysis, the PROPEP nozzle calculations used for an 'idea nozzle’ (ideal expansion) run of this sim appear to use a slightly higher value of gamma for the nozzle exit than that for the throat, due perhaps to imperfect gas assumptions.

The code that follows assumes that gamma remains constant downstream of the throat, and so it turns out that the optimum expansion ratios calculated by PROPEP should be reduced by about one quarter of a percent for a truly Frozen-flow analysis downstream of the throat, which corresponds to a nozzle exit diameter about 4 percent smaller. This could equally be occurring simply due to numerical precision errors.

PROPEP’s C* calculation seems to be using the throat gamma, as the chamber stagnation pressure P0 works out pretty close to PROPEP’s given chamber pressure P, so it should be valid to calculate C* times CF (thrust coefficient) to get the thrust, where I've calculated CF using the throat gamma (see later).  ]

A ‘real’ nozzle (one with a fixed area ratio) is simmed, but this requires an implicit iterative calculation loop to calculate the nozzle exit pressure.

For this, an equation is required to allow recalculation of the nozzle exit pressure using Humble equation 3.98 page 102, and using standard isentropic compressible flow relations (e.g. Humble equations 3.67 to 3.69 page 96) to substitute pressure for temperature times density, and assuming that throat Mach number M* is equal to 1:

ε=AexitA*=M*MexitT* ρ*2Texit ρexit2   =1MexitT* ρ*2Texit ρexit2

=1MexitP*Pexitγ-1γP*Pexit2γ    =1MexitP*Pexitγ+1γ   where ε is the standard nozzle expansion ratio.

hence:  ε Mexit2=P*Pexitγ+1γ   giving:    ε Mexit2γγ+1=P*Pexit

and finally:    Pexit=P*ε Mexit2γγ+1

The iteration loop now proceeds as follows:

- Guess the initial nozzle exit pressure Pexit_guess (using ambient pressure as first guess).

- Calculate nozzle exit Mach number based upon this nozzle exit pressure (as above):
Mexit=2γ-1P0exitPexit_guessγ-1γ

- Re-calculate nozzle exit pressure using the equation above:  Pexit=P*ε Mexit2γγ+1

- If Pexit_guess is less than Pexit then increase Pexit_guess, otherwise decrease Pexit_guess.

- Go back to step 2. and iterate until Pexit_guess is equal to Pexit to a required accuracy.

Continuing with the simulation of a real (fixed) nozzle, a check for nozzle overexpansion is now performed (Humble equation 3.139 page 117).

If the back-pressure (ambient pressure) on the nozzle is too high, the flow expands down the nozzle only so far, before it jumps back up to ambient pressure via a normal shockwave. The effective area ratio when this happens is based on the cross-sectional area of the nozzle just upstream of said shockwave.

Humble et. al.’s criteria for the onset of separated nozzle flow is if the ratio: ambient pressurenozzle exit pressure>2.75

The sim checks whether this criteria has been breached, and if so then the exit pressure is set to this ratio of 2.75. Then the nozzle exit Mach number is recalculated using this corrected pressure ratio in the equation given earlier:

Mexit=2γ-1P0exitPexitseparated_nozzle _flowγ-1γ

The effective nozzle area ratio (just ahead of the shock) is then calculated using the equation given earlier:
ε=1Mexit2γ+11+γ-12Mexit2γ+1γ-1

Next, the nozzle’s pressure thrust is calculated for user output:

Thrustpressure=ηPeAe- PaAe   =ηPe- PaAe=ηPe- Pa ε A*

where η is the combined nozzle efficiency discussed earlier.

Next, a check is performed to make sure that the nozzle is choked. For the lowest chamber pressure in the data tables (2.5 Bar) this shouldn’t be a problem for most propellants (but it would hypothetically if the chamber was full of helium or a similar monatomic gas).

Next, the nozzle thrust coefficient CF is calculated, using Humble equation 3.129 page 112:

CF=2γ2γ-12γ+1γ+1γ-11-PexitP0γ-1γ+εPexit-PambientP0

With CF calculated, the engine thrust can now be calculated:

Thrust=mnozzleC*CFn     where η is the aforementioned nozzle efficiency.

The current Specific impulse (seconds) can also now be calculated:

Isp=Thrustmnozzle g   where g is 9.81

Finally, the vacuum Specific impulse is calculated purely for user output, using the vacuum CF equation. This assumes that both the ambient pressure and nozzle exit pressure are zero (vacuum conditions on an infinitely-expanded nozzle). Humble equ. 3.194 page 140 is used:

CF_vac=γ2γ-12γ+1γ+1γ-1

With the combustion chamber and nozzle both simmed, the program can now update parameters and variables ready for the next time iteration.

Update of parameters

Mass accumulation within the chamber with time - resulting from the imbalance of gas inflow minus gas outflow - can then be modelled using the following unsteady lumped-parameter method of Heister and Landsbaum (See Humble page 338 equation. 6.34).

Note that the ballistic element method (Humble page 342) would be more appropriate if the gas speed down your hybrid fuel port was discovered to be a high fraction of sonic speed (which it shouldn’t be).

The ideal gas law can be rearranged to yield the current mass of gas within the combustion chamber.

m=PVRT

Taking the natural logarithm of both sides of this equation, and using the law of logarithms:

lnm=lnPVRT=lnP+lnV-lnR-ln(T)

Differentiating this using the chain rule, one obtains:

ddm(lnm) dmdt=ddp(lnP)dpdt+ddV(lnV)dVdt-ddR(lnR)Rdt-ddT(lnT)dTdt

giving:

1m dmdt=1pdpdt+1VdVdt

(because the derivative of lnx=1x and neither T nor R change with time: their derivative with time is zero).

Rearranging:

dPdt=P1m dmdt-1VdVdt

1m refers to the (inverse of the) current gas mass within the combustion chamber, and 1V refers to the (inverse of the) current chamber volume. The rate of change of combustion chamber volume dVdt was computed earlier.

This rate of change of chamber pressure is then rate-limited to 300 Bar per second to filter out the chamber oscillations that occur within combustion chambers. Then it’s integrated to get the current chamber pressure. Note that with the stiffer integration now used, this rate-limiting is primarily to avoid an improbably-steep ramp-up of combustion chamber pressure just after engine ignition.

Subroutine Stoichiometric() is now called: this is a routine that scans the PROPEP 3D array looking for the fuel-to-oxidiser ratio that yields the best Specific impulse at the current chamber pressure. Two values are output, the frozen and shifting results.

#### Integrals

Then the user-input nozzle throat erosion rate is integrated to yield the current throat diameter, and throat cross-sectional area.

Thrust is integrated to yield the running Total impulse.

Nozzle mass flow rate is integrated to track the usable propellant that has been burnt.

If the engine type is a hybrid, the fuel port regression rate is integrated to yield the current port/s geometries, and then subroutine Hybrid_port_dimensions() is called to get current port/s areas, volumes, and solid grain mass.

#### Burnout tests

Next, the engine is tested for burnout, which occurs:

- If the liquid propellant masses become zero.

- If the solid fuel grain erodes away completely (zero mass).

- If the propellant combination allows: if the vapour mass remaining in the tanks becomes zero.

- If the combustion chamber pressure falls below 2.5 Bar Abs (or whatever the .propep input file defined as the lowest allowed chamber pressure).

- If the combustion chamber pressure falls sufficiently low to unchoke the engine nozzle.

- If the mass flux down the hybrid fuel port gets excessive and blows the flame out.

- If the ox/fuel ratio wanders out of the data range specified in the .propep input file.

The particular type of burnout occurring is assigned a label that affects the burnout message printed on the sim main window.

If burnout has occurred, engine thrust is set to zero.

Also, (and rather artificially), nozzle mass flow rate, and hot gas mass within the chamber, are also set to zero at the instant of engine burnout, whereas in reality they would ramp down to zero over a few seconds or so. (This is done to make mass calculations easier in the Aspiresim trajectory simulation models, but was worth leaving in.)

The average Specific impulse over the burn-time is now calculated.

The percentages of remaining propellant mass are sent to the fuel and oxidiser displays on the sim main window.

The program now leaves subroutine Simulate_engine(), returning to subroutine EngineModelInterface() within file EngineModelInterface.cpp. This calls subroutine EngineModelOutputs() to output engine variables:

## EngineModelOutputs()

Next, engine information is sent to be printed on the sim main window, and graphed on the engine graphs sub-window.

Next, text output file firing_data.csv is opened, and written-to.

The sim then returns to subroutine EngineModelInterface(), integrates burn_time, and then leaves. This time-iteration of the simulation is now complete.

## Glossary:

Class: a type of Structure (see below).

Combo-box: a drop-down list of files within a GUI.

Constructor: a subroutine that runs first before anything else, denoted by repeating the class name, e.g. Cengines::Cengines()

Destructor: the partner to the Constructor, it runs at the very end. Again, it’s denoted by repeating the class name, but with a tilde symbol e.g. Cengines:: ~Cengines()

Device context: a Windows data structure containing information about the drawing attributes (e.g. size, resolution) of a display. Device contexts allow device-independent drawing.

Dialog box: a window that contains one or more controls. An application uses a dialog box to prompt the user for input needed to complete a command.

Edit box: A small box used to input a single data within a GUI.

Flag: usually a Boolean (true or false) variable used as a trigger for something else.

GUI: Graphical User Interface, the windows and controls used by the user to input data.

Header file: All elements in a program (variables, functions, classes etc) must be declared before they can be used. C++ adopted the convention of making these declarations in a header file, then using the #include directive in every .cpp file that requires those declarations. The #include directive inserts a copy of the header file directly into the .cpp file before it’s compiled.

Radio button: one of several clickable ‘dots’ within a GUI.

Static: In C and C++ programming, a static variable preserves both its contents and its memory location. A static variable is declared using the ‘static’ keyword and retains its value between multiple function/subroutine calls. It is initialised only once and is not destroyed when the function/subroutine returns a value. It extends the lifetime of the variable until the end of the program.

Structure: a data structure containing a list of variables and subroutines.

--------------------------------------------------------------------------------------------------------------------------------------

Critical point: Substances below their Critical point (below their respective Critical temperature and Critical pressure), can exist as more than one physical phase simultaneously, e.g. a liquid coexisting with a vapour. (See our paper “The physics of nitrous oxide” downloadable from the Aspirespace website.)

Compressibility factor: Gasses at low temperature (or high density) such as Nitrous oxide vapour, deviate from ideal gas behaviour. They can be described by introducing a fiddle-factor known as the compressibility factor Z:     P=Z ρ R T

Discharge coefficient: (or ‘efflux coefficient’) In, the ratio of the actual discharge out the end of a nozzle or orifice compared to the ideal discharge. This represents the loss of total pressure (an energy loss) due to viscous losses/turbulence as the flow flows through the edges of the orifice. It depends on the fluid used, and also how sharp the edges of the orifices are:

Cdis=mactualmideal

Frozen flow: The assumption that whatever the mixture of exhaust gas chemicals was at the nozzle entrance, then this mix stays the same during its flow through the nozzle. This is called frozen flow because the mix is ‘frozen’, that is, unchanging.

Ideal gas: a gas that is far above its Critical point and thus obeys the ideal gas equation of state (ideal gas law):

P=ρ R T  where P is the pressure of the gas,  its density, and T is its absolute temperature.

R is the specific gas constant for this gas.

A ‘perfect’ gas may or may not be an ideal gas depending on who you ask!

Mach number: The vehicle’s airspeed V (or the local airspeed around a nose or fin) compared to the speed of sound a of the atmosphere:

M=Va

Real gas: a gas that doesn’t behave as an ideal gas (see Compressibility factor).

Saturated fluid: As an example, a closed container containing a mixture of liquid water and water vapour at 1 atmosphere pressure and 100°C is a saturated fluid, as its temperature and pressure are below water’s Critical point. Similarly for a container of nitrous oxide below its Critical temperature and pressure.

Saturated vapour: a saturated fluid devoid of any liquid, if for example, all the liquid phase has been vaporised or drained off.

Shifting flow: The assumption that the mixture of exhaust gas chemicals is constantly changing during its flow through the nozzle: new molecules form and others break up as the temperature drops down the nozzle. This is called shifting flow because the mix shifts (also known as ‘equilibrium flow’).

Subcritical: Generally taken to mean a substance that is only just below its Critical point. Note that a subcritical gas is termed a ‘vapour’.

Subsonic:  Vehicle airspeed is below Mach 1 (see Mach number).

Supersonic:  Vehicle airspeed is above Mach 1 (see Mach number).

Vapour pressure: the pressure exerted by a subcritical vapour.

## References:

Ref. 1: ‘Space Propulsion Analysis and Design’ by Ronald W. Humble, Gary N. Henry and Wiley J. Larson, McGraw Hill Space Technology Series ISBN 0-07-031320-2, 1996’

Ref. 2: ‘The science and design of the hybrid rocket engine’ by Richard M. Newlands, Lulu Press, 2017

Ref. 3: “Rocket Propulsion Elements 7th edition”, Sutton and Biblarz

Ref. 4: https://en.wikipedia.org/wiki/Perfect_gas

Ref. 5: “Generalised Isentropic Relations in Thermodynamics”, Pim Nederstigt  and Rene Pecnik, Department of Process and Energy, Delft University of Technology, Energies, 16(5), Article 2281. https://doi.org/10.3390/en16052281 2023. See: https://research.tudelft.nl/en/publications/generalised-isentropic-relations-in-thermodynamics

Ref. 6: Engineering Sciences Data Unit (ESDU) sheet 91022, “Thermophysical properties of nitrous oxide.” Available in hardcopy from some U.K. University libraries, or accessible over the Web to students.

Ref.7: https://www.thermopedia.com/content/1107/